Password based door lock (Code)

#include <Keypad.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

Servo myServo;

const byte ROWS = 4;

const byte COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[ROWS] = {9, 8, 7, 6};

byte colPins[COLS] = {5, 4, 3, 2};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

String password = "1234";

String input = "";

int servoPin = 10;

int buzzer = 11;

void setup() {

myServo.attach(servoPin);

pinMode(buzzer, OUTPUT);

lcd.init();

lcd.backlight();

myServo.write(0);  // Locked position

lcd.setCursor(0,0);

lcd.print("Enter Password");

}

void loop() {

char key = keypad.getKey();

if (key) {

lcd.setCursor(input.length(), 1);

lcd.print("*");

input += key;

if (input.length() == 4) {

delay(300);

if (input == password) {

lcd.clear();

lcd.print("Access Granted");

tone(buzzer, 1000, 200);

myServo.write(90);

delay(3000);

myServo.write(0);

}

else {

lcd.clear();

lcd.print("Wrong Password");

tone(buzzer, 500, 500);

delay(2000);

}

input = "";

lcd.clear();

lcd.print("Enter Password");

}

}

}