#include <Keypad.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

Servo myServo;

const String password = "1234";

String input = "";

const byte rows = 4;

const byte cols = 4;

char keys[rows][cols] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[rows] = {9,8,7,6};

byte colPins[cols] = {5,4,3,2};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, rows, cols);

int buzzer = 11;

void setup() {

lcd.init();

lcd.backlight();

myServo.attach(10);

pinMode(buzzer, OUTPUT);

lcd.print("Enter Password");

}

void loop() {

char key = keypad.getKey();

if (key) {

tone(buzzer, 1000, 100);

input += key;

lcd.setCursor(0,1);

lcd.print(input);

if (input.length() == 4) {

delay(500);

if (input == password) {

lcd.clear();

lcd.print("Access Granted");

myServo.write(90);   // unlock

} else {

lcd.clear();

lcd.print("Wrong Password");

tone(buzzer, 500, 500);

myServo.write(0);    // lock

}

delay(2000);

lcd.clear();

lcd.print("Enter Password");

input = "";

}

}

}